How I Illuminated the Non Fairmont style XR6T window switches

NOTE This is just what I have done I am unsure of the warranty implications please make any modifications at your own risk.

2 Single Switch

[image: image1.jpg]



Soldered Bridge

390 Ohm resistor

LED soldered to neg








New Pin

      and its pad

Drivers switch – Where I positioned the LEDs

[image: image2.jpg]



This also shows bridge positions and


the new pin.

Practice on the single switches first.

All leds have a 390 ohm resistor.

The drivers switch comes out and apart the same as the single switch except mine was quite tight removing it from the drivers door.

Positioning the LEDs to get an even spread of  light can be tricky.

Center pin was slipped into place without removing pcb from backing.

LED Connections

[image: image3.jpg]




        Negative
Positive


Required

      Piece of thin aluminium strip for extra plug pin

7 - 700 to 900mcd bright blue LED 3mm

1 – Standard red 3mm LED
8 – Resistors 390 ohm
Heat Shrink

Solder & a small iron

Small flat screwdriver

More connections

[image: image4.jpg]




Negative connections


Keep these wires low

Avoid touching contacting surfaces of the switch contacts

Ensure no wires interfere with contacts or levers and reassemble

[image: image5.jpg]



Finished- lit up at night

PAGE  
5

