	Cleaning the Idle Speed Controller



	The Idle Speed Controller Valve, or otherwise known as the Idle Air Bypass System, is responsible for maintaining a smooth, and consistent idle, under different Engine Loads, and temperatures.
It does this by allowing a certain amount of air, to bypass the Throttle Body Butterfly, thereby increasing the amount of air entering the engine, under certain conditions, thereby increasing Engine RPM.
The Idle speed control valve is controlled by the EEC-V Engine Management system, and is a spring loaded stepped motor, that opens a small valve, that allows air to flow. It is actually quite common for this valve to get gummed up and slighty jammed, by carbon deposits, oil, and other foreign materials.
This is especially the case if an aftermarket air cleaner, that is oil based (Such as K & N air Filters), as the Oil is slowly sucked in to the Idle speed controller, and slowly blocks off, and jams the air passage and Valve.
Symptoms of a jammed, or dirty Idle Speed Controller include, 

  1) Poor idle quality in cold temperatures. 

  2) Engine stalling under load (Eg Power steering operation, Air Conditioning, etc), under closed throttle (Idle) conditions. 

  3) Rough Gear changes on Automatics when changing from second, to first gear, (Engine speed surging). 

  4) Engine RPM surging at idle, for no reason. (I.E. Engine increasing RPM to greater than 1000rpm, for no apparent reason).

Cleaning the Idle Speed Controller
The Idle speed controller can be easily cleaned, and is recommended if the above symptoms exist.
To remove the Idle Speed controller, you will need an 8mm wrench, or an adjustable wrench.

  1. Disconnect the Electrical Connector, by pulling down on the bottom Plastic latch, on the Bottom of the connector. 

  2. Remove the 2 8mm Bolts that hold the Idle Speed Controller in place. 

  3. Remove the Idle Speed Controller.

The Following Images will give you some idea of what needs to be cleaned, with regards to the ISC Valve.

	[image: image1.jpg]




 INCLUDEPICTURE "http://www.fordmods.com/Images/ISC.jpg" \* MERGEFORMATINET [image: image2.jpg]




	From the above picture, you can see the Plunger, that is used to control the amount of air that is to be passed. Also you can see the two air passageways. The air passageways need to be cleaned, and this can be achieved with some Turps, and by rubbing the passageways with your finger.
Note: Be carefull if you are going to use a rag or such to clean the passageways, as any material that may be left or stuck in the passageways will be sucked in to the engine, causing damage !
With The ISC Valve, the Best thing to do is fill the Passageways with turps, or WD40, then cover the holes, and shake thoroughly, to ensure that both the plunger, and passageways have been cleaned.
Note: the liquid will flow through to the Black Breather cap when shaking.
Once the plunger and passage ways have been cleaned, then Dry the assembly, and re-install the ISC on to the Throttle Body.


