AU1 radio Connector (C-186)

Pin 1 Yellow Constant Battery Voltage (+ve Battery) 

Pin 2 Brown Button Backlighting (Park Lamps power) 

Pin 3 Grey / Dark Green -ve Speaker - Right Front Speakers 

Pin 4 Orange / Light Green -ve Speaker - Left Front Speakers

Pin 5 Dark Blue / Red +ve Speaker - Right Rear Speaker 

Pin 6 Pink / Dark Green +ve Speaker - Left Rear Speaker 

Pin 7 Dark Blue / Red LCD Backlight and Dimming Signal (-ve) 

Pin 8 Black Negative to Chassis

Pin 9 Violet / Red Accessories Power (Acc on Ignition) 

Pin 10 not connected

Pin 11 White Speedo signal (Hi Series variable volume)

Pin 12 Grey / Black +ve Speaker - Right Front Speakers 

Pin 13 Red / Dark Blue Power Antenna Output (Positive Trigger) 

Pin 14 Orange / Black +ve Speaker - Left Front Speakers

Pin 15 Dark Blue/Black -ve Speaker - Right Rear Speaker 

Pin 16 Light Green Telephone Mute (Mobile phone Car Kit)

Pin 17 Pink / Black -ve Speaker - Left Rear Speaker

Pin 18 Red / White Steering wheel controls

Pin 19 Pink Mobile Phone Audio In

Pin 20 Light Blue / Yellow Mobile Phone Audio Signal Ground


 INCLUDEPICTURE "http://www.fordforums.com.au/pub_images/AUAudioCa.gif" \* MERGEFORMATINET 




Steering Wheel Control Buttons - all AU models (C-166)







AU2/AU3 Radio Connector (C-186)

Pin 1 Yellow Constant Battery Voltage (+ve Battery) 

Pin 2 Brown Button Backlighting (Park Lamps power) 

Pin 3 Grey / Dark Green -ve Speaker - Right Front Speakers 

Pin 4 Orange / Light Green -ve Speaker - Left Front Speakers

Pin 5 Dark Blue / Red +ve Speaker - Right Rear Speaker 

Pin 6 Pink / Dark Green +ve Speaker - Left Rear Speaker 

Pin 7 Dark Blue / Red LCD Backlight and Dimming Signal (-ve) 

Pin 8 Black Negative to Chassis

Pin 9 Violet / Red Accessories Power (Acc on Ignition) 

Pin 10 not connected

Pin 11 White Speedo signal (Hi Series variable volume)

Pin 12 Grey / Black +ve Speaker - Right Front Speakers 

Pin 13 Red / Dark Blue Power Antenna Output (Positive Trigger) 

Pin 14 Orange / Black +ve Speaker - Left Front Speaker s

Pin 15 Dark Blue/Black -ve Speaker - Right Rear Speaker 

Pin 16 not connected

Pin 17 Pink / Black -ve Speaker - Left Rear Speaker

Pin 18 Light Blue / White Diagnostics

Pin 19 Red / Black External Controls (Data Link ?)

Pin 20 Orange Black External Controls (Data Link ?)






AU2/AU3 Radio Connector (C-232)



Pin 1 Red SubWoofer Control

Pin 2 Yellow SubWoofer +ve Audio

Pin 3 

Pin 4 

Pin 5 Light Green Telephone Mute (Mobile phone Car Kit)

Pin 6 Pink Mobile Phone Audio In

Pin 7 Heatshrink SubWoofer Cable Shield/Return

Pin 8 Heatshrink SubWoofer –ve Audio

Pin 9 

Pin 10 

Pin 11 

Pin 12 Light Blue / Yellow Mobile Phone Audio Signal Ground

Pin ?? Dark Green / White Navigation - Audio

Pin ?? Pink / Orange Navigation – Signal Ground

Pin ?? Dark Blue Navigation- Mute


