Falcon/Fairmont BA Airbag & Steering Wheel Removal Procedure
Special tools required,

T-30 & T-50 Torx bits and driver.

Steering Wheel Puller (recommended) 
1. Disabling the Airbag system, 

This can be achieved by either disconnecting the battery for 5 minutes prior to commencing this procedure or removing the airbag fuse (fuse #3 7.5amps inside the instrument panel fuse box.)
2. Removing the two air bag retaining screws,

To access these two screws the steering wheel needs to be rotated 90º from the Horizontal (as shown in picture) at this point only the top screw can be accessed. Using a T30 Torx bit loosen this screw. (Note the screw does not come all the way out of the wheel.) Rotate the wheel 180º to get access to second screw, loosen this one also.

[image: image1.jpg]


 [image: image2.jpg]



3. Removing the Airbag,
Carefully position airbag assembly to gain access to the wiring, see photo.

Disconnect wiring from steering wheel buttons (large white plug). 

Carefully disconnect the two airbag connectors from back of airbag assembly. (1 x grey, 1 x black) 
Position removed airbag to a safe place, i.e. back seat. 
[image: image3.jpg]


 [image: image4.jpg]Airbag power c QG ‘ ;





Removing wheel without a steering wheel puller,
Using a T-50 Torx bit loosen several turns but do not remove large bolt from centre of steering wheel. (Removing this bolt completely may cause personal injury when not using a steering wheel puller.)
Using both hands, grasp the rear of the wheel near the spokes. With both feet braced on the floor, rock, pull, heave on wheel until it is loose on the shaft. 
Unscrew T-50 bolt and remove wheel while carefully threading wires through access hole in wheel.

4. Removing wheel using a steering wheel puller,
Using a T-50 Torx bit loosen several turns but do not remove large bolt from centre of steering wheel. (Removing this bolt completely may cause damage to the steering shaft thread when using a steering wheel puller). Set puller up using the two threaded holes on either side of the T-50 bolt. 
Tighten the pullers centre bolt down onto head of T-50 bolt until wheel comes loose. Remove puller from wheel, then unscrew T-50 bolt and remove wheel while carefully threading wires through access hole in wheel.

Procedure written by Madmax for www.fordmods.com

